Toaster

USER’S MANUAL

Thank you for purchasing this product.

In order to making full use of functions of this product and minimize unnecessary loss and damage, please read the manual carefully and keep it for future reference.

IMPORTANT! Keep these instructions for future reference. If the unit changes ownership, be sure this manual accompanies the equipment.

WARNING! Any refitting and wrong installation, adjustment and maintenance can lead to property loss and injuries. Please contact the supplier if you need to adjust or maintain it and all these should be done by authorized professionals.

WARNING! Bread will be burned, so please do not put this machine near or under curtain or other flammable objects. User should watch this machine working.

# Basic parameter

| Model | HET-4 | HET-6 |
| --- | --- | --- |
| Rating voltage | 230V/50Hz | 230V/50Hz |
| Rating power | 2240W | 3240W |
| Timer range | 0~5min | 0~5min |
| Working election | 2 or 4 slices | 4 or 6 slices |
| Dimension(mm) | 330×220×215 | 420×220×215 |

# Caution of installation

  - The voltage of this product should be the same with the voltage of the power supply.

  - User should install a proper three-phase power outlet and a fuse.

  - Three-phase outlet must be well connected with the ground to ensure safety for electricity using.

WARNING! If you put this machine near wall, kitchen equipments or describing painting objects, please

# Instruction

  - Working selecting switch is printed (- =). When you put switch in (-) position only half of slots work; when you put switch in (- =) position all slots work.

  - Plug the machine on a power socket with voltage the same as rated voltage of this machine. Adjust working selecting switch and put bread into slots, rotate timer clockwise to preset the time. Informed by a “Ting” sound when the bread heating is finished. Press Ejecting Switch and the bread is leaped out.

  - During the heating process, crumbs of bread will fall down on the tray and should be cleaned at once.

  - The size of the bread slice for heating must be bigger than the half of length of the whole slice and if it’s too small, ejecting may be difficult.

  - You should stop using this machine at once while unwanted things happen during using period.

  - If the power cord is damaged it should be replaced by professional workers from manufacturer or service department.

Warning! Do not touch the shell of this machine during using period in case of scald or other injury caused by the high temperature.

# Maintenance

  - Before cleaning this machine you should cut off power supply and unplug it.

  - After work, please pull the food tray out to clean and swear body of the machine with a wet towel. Do not put the whole machine into water to clean or clean it with a water jet in case of damaging machine electric parts.

  - If you do not use this machine, please turn off power supply and unplug the machine.

  - If you stop using this machine for a long time you should carefully clean whole machine and put it in a ventilated place without contact with any corrosive gas or humid environment.

Warning! Do not clean this machine with a water jet.

# Transportation and storage

During transportation, the machine should be careful handled and prevent from shaking. The packaged machine should not be stored in open air for a long period of time. It should be put in a ventilated warehouse without contact with corrosive gas. And it should not be put up side down. If it needs to be stored in open air temperately, protection against rain is necessary.